| Noble Huang (Mulia Widjaja) |  |  |
| --- | --- | --- |
| SANTA CLARA UNIVERSITY | ELEN 115 – Spring 2023 | S. Krishnan |
|  | Homework #6 |  |

5. The regulator in Figure 5 employs a zener diode DZ that is specified to  have a 8V drop at a test current of 25mA with rz =10Ω and IZK =0.2mA.

- Find the value of R needed to obtain an output voltage Vout = 7.8V at nominal supply voltage Vs and no load.

D

Z

R

Vs = 10V

±

15%

V

out

With the value of R as obtained in (a) and nominal Vs find the value of Vout with a load resistance of

  - RL= 10kΩ

  - RL= 1kΩ